# MO-03 Water Reservoir Execution Result v1.0

PV-PP MO-Gymnasium Public Benchmark Program | August 2026

# Execution result

PASS - joint-adequacy infeasibility and state-dependent fallback boundary demonstrated. Automated regression: 11 passed, 0 failed.

# MO-03A - Exact four-domain infeasibility

The decisive contradiction is independent of inflow, reservoir level, and hydroelectric production. In the source equations, zero water-supply deficit requires release >= 50. Zero downstream flooding cost requires release <= 30. Both conditions cannot be true for the same action.

Therefore the strict four-domain zero-deficit adequate set is empty. This is not a poor optimum; it is structural infeasibility under the frozen framing. Adding the upstream and hydroelectric requirements cannot repair the contradiction.

# MO-03B - State-dependent safety boundary

For a controlled deterministic slice using the source mean inflow of 40, zero upstream flooding requires next reservoir level <= 50, while zero downstream flooding requires release <= 30. With next level = state + 40 - release, the two safety conditions can coexist only while state <= 40.

| Initial level | Safety-adequate actions | Safety interval | Equal-weight scalar action | All-4 adequate? |
| --- | --- | --- | --- | --- |
| 20 | 41 | 10 to 20 | 20 | No |
| 40 | 1 | 30 to 30 | 40 | No |
| 60 | 0 | None | 50 | No |
| 100 | 0 | None | 50 | No |
| 140 | 0 | None | 50 | No |

At level 20, releases from 10 through 20 satisfy both safety conditions in the mean-inflow slice. At level 40, release 30 is uniquely safety-adequate. At levels 60, 100, and 140, the safety-adequate set is empty. The same framed problem therefore crosses from an ordinary adequate-policy region into an unavoidable tradeoff/fallback region as state changes.

# MO-03C - Scalar comparator

An equal-weight linear scalar objective nevertheless returns a maximizing action at every tested state: 20, 40, 50, 50, and 50 for initial levels 20, 40, 60, 100, and 140. At level 60, for example, release 50 eliminates the water deficit and upstream flood cost in the mean-inflow slice, but incurs a downstream flooding cost of -20.

The scalar winner is a legitimate compromise under that weighting. What scalar maximization alone does not express is the separate structural statement that the four-domain adequate set is empty, or that the safety-only adequate set disappears above the state boundary. That distinction must be supplied by constraints/adequacy logic or equivalent external structure.

# PV-PP interpretation

The current canonical adequacy operator is conjunctive across governing domains and prohibits cross-domain compensation. It also preserves the empty-set condition when no jointly sustainable policy exists. MO-03 therefore maps naturally onto the existing architecture: the framework does not need to pretend that the numerically best compromise restores all governing domains.

# What this establishes

The public Water Reservoir mechanics contain an exact conflict between full water-demand satisfaction and zero downstream flood release.

The presence or absence of a jointly safe action changes with reservoir state under the controlled mean-inflow slice.

A linear scalar objective still produces an argmax even when the explicitly framed conjunctive adequate set is empty.

No new PV-PP operator, state primitive, or architecture change is required.

# What this does not establish

It does not prove that scalar methods cannot be augmented with constraints, thresholds, lexicographic rules, or fallback flags.

It does not identify a universally correct public policy for reservoir management.

It does not compare learning speed or policy quality against MORL algorithms.

The state-boundary probe uses mean inflow for transparency; the actual environment has stochastic Gaussian inflow, so probabilistic safety requires an explicit risk tolerance.