# MO-14 Highway Result v1.0

PV-PP MO-Gymnasium Complete-Suite Coverage Program | Miscellaneous Phase | August 2026

# Execution result

PASS - safety adequacy and source-scalar mapping boundary demonstrated. Regression: 14 passed, 0 failed.

# Reward structure

Both current registrations expose [high speed, right lane, -collision], bounded [0,1] x [0,1] x [-1,0], and gate the vector to zero when the vehicle is off road. The underlying default scalar uses raw reward -collision + 0.1(right lane) + 0.4(high speed), then normalizes the interval [-1, 0.5] to [0,1], then applies the same on-road gate.

# A narrow scalar-mapping boundary

A pure fixed linear map from the exposed 3D vector cannot reproduce the legacy scalar globally. An on-road state with speed=0, right-lane=0, collision=0 exposes vector [0,0,0] but has normalized scalar reward 2/3. An off-road state is also exposed as [0,0,0] but has scalar reward 0. One identical vector therefore corresponds to two different legacy scalar values. An affine map plus the on-road indicator can reconstruct the source formula; the result is not a claim that scalar representation is impossible.

# Collision and adequacy

The source normalization can leave a crash transition with a positive scalar value. A fast right-lane crash profile [1,1,-1] yields normalized legacy reward 1/3, even though collision terminates the episode. With a deliberately weak collision weight [0.8,0.2,0.05], that crash profile also outranks a safe mid-speed right-lane profile [0.5,1,0]. This is a preference construction, not a defect in weighted sums.

Under the controlled PV-PP framing "remain collision-free while making productive progress," collision is instead an adequacy condition. Crash-bearing candidates are excluded before speed/lane selection. This preserves the distinction between terminal safety failure and ordinary performance tradeoffs without adding a new operator.

# HighwayFast equivalence

HighwayFast materially changes execution conditions - fewer lanes and vehicles, shorter duration, lower simulation frequency - but inherits the same reward semantics. For the present reward/adequacy question it is equivalence-covered at the shared-source level. A future traffic-dynamics study could still treat it separately if stochastic trajectory behavior became the object of the benchmark.

# Disposition

Primary class: A - collision/terminal safety adequacy.

Secondary finding: pure fixed-linear recovery of the legacy scalar from the exposed vector is globally impossible because the on-road gate is omitted from that vector.

mo-highway-v0: COVERED/PASS at source-mechanism level.

mo-highway-fast-v0: COVERED/PASS by shared reward semantics plus configuration reconciliation.

Canonical architecture change: NONE. Formalization gap: NONE. Architecture failure: NONE.

Fidelity limit: no full highway-env stochastic traffic rollout.